Overall Ground Capacity Figures as of 26th March 2021

Entry Capacity: 5280

Exit Capacity: 5051

Standing and Seated Capacity of Stadium: 4085

Therefore, current maximum capacity of the stadium based upon the capacity calculated figure (Exit Capacity) would be 4085.

NOTES

For the purposes of calculation, any calculated decimal figure regarding the number of people in any area, or for egress capacity, has been rounded to the nearest whole number.

Standing capacity calculated by using the formula of 4.7 persons per square metre.

The figure and calculations used in this document have been in accordance to those set out in the 6th Edition of the “Green Guide” – Guide to Safety at Sports Grounds.

Ground Entry Capacity

8 off Turnstiles rated at 660 users per hour each, flat and sound concrete internally and externally.

One block of 4 located in the North West Corner of the ground, one block of 2 located on the North side adjacent to Exit C and one block further north on the same side adjacent to exit D.

P=1 S=1

8 x 660 = 5280

Ground Exit Capacity

5 exit gates designated A to E, located around the ground of various dimensions, either push bar operated or operated by a designated steward when the stadium is occupied. Calculations are based upon a maximum of 82 people per minute per metre width, with a maximum exit time of 8 minutes.

Gate A – (Main gate) A pair of wrought iron gates located adjacent to the turnstile block manned by a designated steward during time the stadium is occupied. Flat, sound concrete internally and externally to the fire assembly area in the site car park. 3.2m wide and with sufficient clearance for emergency vehicles to enter the ground.

P=1 S=1

3.2 x 82 x 8 x 1 = 2099

Gate B – (Between the Brian Collins stand and the club house) A pair of timber gates located near to the entrance of the social club bar, operated by a push bar. Flat, sound concrete internally and external path leading to the fire assembly area minimum 1.6m wide but exiting to 1.5m wide when both gates open.

P=1 S=1

1.5 x 82 x 8 x 1 = 984

Gate C – (Half-way line tractor entrance) A single aluminium gate hinged right to left located 20m north easterly from the pitch half way line in the northern perimeter wall manned by a designated steward during time the stadium is occupied. Flat sound concrete internally leading to flattened tarmac base externally for a distance of 15m before abutting concrete roadway to fire assembly area. 3.7m wide and with sufficient clearance for emergency vehicles to enter the ground.

P=1 S=1

3.7 x 82 x 8 x 1 = 2427

Gate D – (Burger bar exit) A metal gate 2.0m wide located in the north easterly corner of the stadium to the right rear of the catering trailer, operated by a push bar.  Flat, sound concrete internally and externally leading to a 2.0m wide type 2 gravel path leads to the external concrete roadway to fire assembly area. 2.0m wide. The walkway to the exit is constrained by a one metre wide concrete pathway.

P=1 S=1

1.0 x 82 x 8 x 1 = 656

Gate E – (between seated stand and Couch Corner) A solid metal gate 2.0m wide located in the easterly side of the stadium, operated by a push catch.  Flat, sound concrete internally and externally leading to a 2.0m wide path with a mixed concrete and type 2 gravel path to the rear of the new eastern covered seated stand. The path leads to the external concrete roadway to fire assembly area its 2.0m wide and exit is via the crash gate to the rear of the catering trailer. There is also a 1.0m lockable gate opposite gate E, the stewards will have the code to unlock this door.

Both Gates D and E flow into the same 2.0m wide path, exit calculations for these gates are combined and the combined gate width of 3.0m is reduced to 2.0m as constrained by the path.

P=1 S=1

2 x 82 x 8 x 1 = 1312

Combined Total Stadium Exit Capacity 2033+984+2427+656+1312 = 7412

“Worst Case Scenario” Combined Exit Capacities (where 1 is inoperable)

A+B+C+D

2099+984+2427+656 = 6166

A+B+D+E

2099+984+656+1312 = 5051

A+C+D+E

2099+2427+656+1312 = 6494

A+B+C+E

2099+984+2427+1312 = 6822

B+C+D+E

984+2427+656+1312= 5379

Therefore, calculated stadium exit capacity based up the scenario where Gate C is inoperable due to an emergency situation will be 5051.

Standing and Seated Capacity of Stadium

For ease of identification the sides of the ground will be indicated thus:

North: the side of the stadium where the social club, tea bar and club shop are situated, contains both seated and standing areas, part of which are covered.

South: the side of the stadium where the main Wenzel stand is situated, which contains both seated and standing areas.

East: the side of the stadium where the “gun turret” is situated which contains both seated and standing areas, part of which are covered.

West: the side of the ground where the changing rooms, tea bar and other ancillary buildings are located which contain only standing areas, part of which are covered in the Holywell Stand and a new stand which is similar in design to the Holywell stand but slightly smaller.

Total Stadium Seated Capacity

75+254+369 = 698

Total Stadium Covered Standing Capacity

142+0+329+293+230 = 994

Total Stadium Uncovered Standing Capacity

758+667+267+701 = 2393

Total Stadium Capacity

698+994+2393 = 4085

North Side = 975

South Side = 921

East Side = 965

West Side = 1224

Total Capacity of the stadium = 4085

The North side

Seated Capacity

North - Elmslie Stand – The secondary seating area in the stadium, covered seating area, Concrete steps with fixed “flip up” plastic seats and steel sheet cladding to roof, with internal incandescent lighting.

3 off rows of seats split into 2 blocks of 8 seats and 12 seats between exit points and gangways.

1 off row of 15 seats along the back most row of the stand.

P=1 S=1

3 x 20 x 1 = 60

1 x 15 x 1 = 15

Total 75

Brian Collins Covered Standing Capacity

North – 3 off flat and sound concrete steps with crush barriers, Varying lengths and depths with steel cladding to roof and rear.

P=1 S=1

9 x 1 x 4.7 x 1 = 42

9.3 x 1.1 x 4.7 x 1 = 48

9.3 x 1.2 x 4.7 x 1 = 52

Total 142

Uncovered Standing Capacity

North

Hawthorn Terrace

A - 2 off flat and sound concrete steps, rearward step with crush barrier, far easterly end between the floodlight pylon and Gate D.

Front step 16m long at a depth of 1m.

Rear step 13m long at a depth of 1.8m.

P=1 S=1

14 x 1.0 x 4.7 x 1 = 66

16 x 1.3 x 4.7 x 1 = 98

16 x 1.0 x 4.7 x 0.5 = 38

Total 202

Telephone Mast Gates

B – 1 off flat and sound concrete step, directly in front of the floodlight pylon near to Gate C.

Step 5m long at a depth of 1.5m. Reduced P factor to 0.3 due to narrow nature of terrace creating a potential bottleneck.

P=0.3 S=1

5 x 1.3 x 4.7 x 0.3 = 9

Total 9

Wheelchair Terrace

C - 2 off flat and sound concrete steps, rearward step with crush barrier, between the floodlight pylon and Gate C.

Front step 7.85m long at a depth of 1m.

Rear step 6m long at a depth of 1.9m only 1.5m used in capacity calculation.

P=1 S=1

7.85 x 1.0 x 4.7 x 0.5 = 12 – Carers 12

7.4 x 1.5 x 4.7 x 1 = 8 -- Wheelchairs 8

Total = 20

Halfway Line Terrace Gate

D - 1 off flat and sound concrete step, directly in front of Gate C.

Step 3m long at a depth of 1.5m. Now a hatched non-standing area

P=1 S=1

Total 0

Club Shop Terrace

E - 3 off flat and sound concrete steps, both steps with crush barriers, between Gate C and Pitch side dug-outs. The front row is reduced P factor to 0.5 due to narrow nature of terrace creating a potential bottleneck.

Walkway 15m long at a depth of 1m.

Front 15m long at a depth of 1.5m.

Rear step 15m long at a depth of 1.5m.

P=1 S=1

15 x 1.0 x 4.7 x 0.5 = 35

15 x 1.5 x 4.7 x 1 = 105

15 x 1.5 x 4.7 x 1 = 105

Total 245

Half way line Tea Bar terraces

F1 - 1 off flat and sound concrete step, between Pitch side dug-outs. Reduced P factor to 0.3 due to narrow nature of terrace creating a potential bottleneck.

Step 3m long at a depth of 1.5m.

P=1 S=0.3

3 x 1.5 x 4.7 x 0.3 = 6

Total 6

Behind Pitch side dugouts

F2 - 1 off flat and sound concrete step with crush barrier, behind Pitch side dug-outs.

Step 6m long at a depth of 2.0m only 1.5 m used.

P=1 S=1

6 x 1.50 x 4.7 x 1 = 42

Total 42

Ground Toilet Terrace

G - 1 off flat and sound concrete step, between Pitch side dug-outs and seated stand.

Step 4m long at a depth of 1.5m.

P=1 S=1

4.4 x 1.5 x 4.7 x 1 = 31

Total 31

Bar floodlight gate area

H1 - 1 off flat and sound concrete step with crush barrier, between seated stand and floodlight pylon.

Step 5m long at a depth of 1.5m. Reduced P factor to 0.5 due to narrow nature of terrace creating a potential bottleneck.

P=1 S=0.5

5 x 1.5 x 4.7 x 0.5 = 17

Total 17

Leo Morris Terrace

H2 – 2 Off step with crash barrier on the rear step

P=1 S=1

4.8 x 1.3 x 4.7 x 1 = 29

4.8 x 0.86 x 4.7 x 1 = 19

Total 48

I -2 off uncovered steps and the pitch side walkway between the turnstile block 1-4 and the floodlight segregation gate

I - 2 off flat and sound concrete steps, rearward step with crush barriers, between floodlight pylon and covered standing are, and to the front of standing covered area. Includes one 8m long ramped section from front step up to rearward step due to pitch side fence being below 1.1m high along its entire length. Reduced P factor to 0.5 due to narrow nature of terrace creating a potential bottleneck.

Front step 17.7m long at a depth of 1.05m.

Rear step 8m long at a depth of 0.75m.

P=1 S=1 unless stated

17.7 x 1.05 x 4.7 x 0.5 = 44 (walkway)

15 x 0.75 x 4.7 x 1 = 53

5.75 x 1.5 x 4.7 x 1 = 41

Total 138

Total North Side Uncovered Standing Capacity

202+9+20+0+245+6+42+31+17+48+138 = 758

Total covered seating area = 75

Total covered standing area = 142

Total North side capacity 758+75+142 = 975

The South side – The Wenzel’s stand covered seating areas

South Stand – The Main Stand in the stadium, covered seating area. Concrete steps with fixed “flip up” plastic seats and steel sheet cladding to roof and sides, with internal fluorescent lighting.

4 off rows of seats split into 2 blocks of 24 seats and 26 seats between exit points and gangways.

1 off row of 54 seats along the back most row of the stand, 6 of which are designated press seats located in a self-contained “press box”

P=1 S=1

4 x 50 x 1 = 200

1 x 54 x 1 = 54

Total 254

Covered Standing Capacity

South – No covered standing areas

South Uncovered Standing Capacity

Dead Fox corner standing areas

A - 1 off flat and sound concrete paved step, far westerly end between the external access gate in the south west corner of the stadium and the main seated stand, passing in front of the floodlight pylon.

Step 10m long at a depth of 1m and 6m at a depth of 1.5 m and 20m at a depth of 2.2m of which 0.7m are used as a path.

P=1 S=1

10 x 1.0 x 4.7 x 1 = 47

6 x 1.5 x 4.7 x 1 = 42

20 x 1.5 x 4.7 x 1 = 141

Total 230

Flat standing areas between the Gun Turret and the Wenzel’s stand

B – 1 off flat and sound concrete paved step, far Easterly end between the “gun turret” in the south east corner of the stadium and the main seated stand, passing in front of the floodlight pylon, which varies in width along its length.

Step 21m long at a depth of 2.2m, 0.7 m are used as path.

Step 19m long at a depth of 1.5m.

P=1 S=1

21 x 1.5 x 4.7 x 1 = 148

19 x 1.5 x 4.7 x 1 = 134

Total 282

C - New Terrace

12 x 1.5 x 4.7 x 1 = 84

10 x 1.5 x4.7 x 1 = 71

Total 155

Total South Side Uncovered Standing Capacity

230 + 282 + 155 = 667

Total covered seating area = 254

Total covered standing area = 0

Total South side capacity 667+254+0 = 921

The East Side

The King’s stand covered seated area

East Stand – The new stand in the stadium, covered seating area. Metal steps with fixed “flip up” plastic seats and steel sheet cladding to roof. .

5 off rows of seats split into 3 blocks of 31 seats between exit gangways.

1 off row of 69 seats along the back most row of the stand, 6 of which are designated press seats located in a self-contained “press box”

P=1 S=1

5 x 60 x 1 = 300

1 x 69 x 1 = 69

Total 369

Couch Corner East Covered standing area

East A – 3 off flat and sound concrete steps with crush barriers, with steel cladding to roof and concrete fencing to rear.  Front most step 20.1m long at a depth of 1.1m, Rear most step at varying depths.

P=1 S=1

20.1 x 1.1 x 4.7 x 1 = 104

20.4 x 1.1 x 4.7 x 1 = 105

17 x 1.5 x 4.7 x 1 = 120 (with crash barrier to the front)

Total = 329

Couch Corner East Uncovered standing area

Corner flag to the end of the grass in front of the gun turret

A1 - 1 off flat and sound concrete paved step, far southerly end between the south eastern corner of the stadium and the raised terrace area adjacent to the “gun turret”.

Step 7m long at a depth of 1.5m.

P=1 S=1

7 x 1.5 x 4.7 x 1 = 49

Total 49

Step and walkway to the south side of the segregation gate in front of the gun turret

A2– 2 steps to the south of the Couch Corner stand 2m in length and 1.5m in depth

P=1 S=1

2 x 1.5 x 4.7 x 1 = 14 Walkway

2 x 1.5 x 4.7 x 1 = 14 Rear step

Total 28

Steps in front of covered area in Couch corner

B – 2 off flat and sound concrete step between the “gun turret” and Gate E to the front of the covered terrace, 21m long at a depth of 1.0m.

P=1 S=1

20.2 x 1.0 x 4.7 x 1 = 95

20.2 x 1.0 x 4.7 x 0.5 =48

Total 143

C- Burger bar Terrace

C - 2 off flat and sound concrete steps with crush barrier to rearward step, between the covered seating area and Gate D.

Front step 5m long at a depth of 1.0m. Rear step 5m long at a depth of 1.5m.

P=0.5 S=1 on the front step

P=1 S=1 on the rear step

5 x 1 x 4.7 x 0.5 = 12

5 x 1.5 x 4.7 x 1 = 35

Total 47

East Side Total Covered Seating capacity = 369

East Side Total Covered standing area =329

East Total Side Uncovered Standing Capacity

49+28+143+47   = 267

Total East Capacity

369+329+267 = 965

The West Side

West Covered seating area = 0

West Covered Holy well standing area

West 1 – 7 off flat and sound steel steps, 25m long at a depth of 2.5m with steel cladding to roof, with crush barriers at front row and between 3rd and 4th row.

P=1 S=1

25 x 2.5 x 4.7 x 1 = 293

Total 293 = 293

West 2 – 7 off flat and sound steel steps, 20m long at a depth of 2.5m with steel cladding to roof, with crush barriers at front row and between 3rd and 4th row.

P=1 S=1

20 x 2.5 x 4.7 x 1 = 230

Total 230 = 230

West Uncovered standing area

A -Turnstile to players tunnel area

A - 1 off flat and sound concrete paved step, far northerly end between the turnstile block and the player’s tunnel.

Step 8m long at a depth of 1.5m.

P=1 S=1

8 x 1.5 x 4.7 x 1 = 56

Total 56

B1 - Players tunnel to the Captain’s lounge steps in front of Ref’s room

B1 – 1 off flat and sound concrete step partly with crush barrier to a depth of 2.2m only 1.5m is considered for capacity between the player’s tunnel and the covered standing area, discounting the yellow hatched area directly at the top of the stepped access. .

Step 6m long.

P=1 S=1

6.8 x 1.5 x 4.7 x 1 = 42

Total 48

B2 – Tarmac path between player’s tunnel and the Captain’s Lounge steps on the referee’s room side of the hatched markings

B2 – Tarmac path between player’s tunnel and the start of the Captain’s Lounge steps to a depth 1.4 metres is considered for capacity.

P=1 s=1

10 x 1.4 x 4.7 x 1 = 66

Total 66

B3 – Tarmac path between player’s tunnel and the steps before the start of the Hollywell stand adjacent to the pitch side of the hatched markings

B3 – Tarmac path between player’s tunnel and the start of the Captain’s lounge steps to a depth of 1m next to the pitch side perimeter fence.

P=1 S=1

10 x 1.5 x 4.7 x 1 = 71

Total 71

B4 – Steps in front of the Captains lounge starting from step 3

B4 – 3 steps two at a depth of 0.33 of a metre and the rear step at 1.5 m

P=1 S=1

5 x 0.33 x 4.7 x 1 = 8 (3rd step)

5 x 0.33 x 4.7 x 1 = 8 (4th step)

5 x 1.5 x 4.7 x 1 = 35 (top step)

Total 51

C1-Stepped standing area in front of the Hollywell stand

3 steps at depths of 0.53m and 2 at 0.35m directly in front of covered standing to the south side of the stadium

P=1 S=1

31 x 0.53 x 4.7 x 1 = 77 (step 3)

29 x 0.35 x 4.7 x 1 = 48 (step 2)

15 x 0.35 x 4.7 x 1 = 25 (step 1 southern end only)

Total 150

Flat standing area by the perimeter pitch fence in front of the Hollywell stand

C2 - Flat standing area by the perimeter pitch fence in front of the Hollywell Stand

9.7 x 0.6 x 4.7 x 1 = 27 (1st Oblong)

2.35 x 0.425 x 4.7 x 1 = 6 (1st Triangle)

2.8 x 1.2 x 4.7 x 1 = 16 (2nd Oblong)

9.1 x 0.9 x 4.7 x 1 = 39 (3rd Oblong)

2.3 x 0.45 x 4.7 x 1 = 6 (2nd Triangle)

4.2 Hatching = 0

Total 94

D -Flat standing area between the end of the Hollywell stand and Dead fox corner

D - 1 off flat and sound concrete paved step, between the area directly in front of the Hollywell stand and the south westerly corner of the stadium, with a large grassed area to the rear.

Step 23.5m long at a depth of 2.0m with only 1.5 considered in the capacity calculation.

P=1 S=1

23.5 x 1.5 x 4.7 x 1 = 165

Total 165

Total West side Covered Seating Capacity = 0

Total West side Covered Standing Capacity

293 + 230 = 523

Total West side Uncovered Standing Capacity

56+48+66+71+51+150+94+165 = 701

Total West side Capacity 523+ 701 = 1224

North Side = 975

South Side = 921

East Side = 965

West Side = 1224

Total Capacity of the stadium = 4085

Measurements and calculations provided by NJ Symmons were checked on a site visit with a physical measurement check and calculation verification. This took place on the 26th March 2021

Signed ..........................................................................................................

Ross Forsyth

Regulatory Services Officer

Resident Services

26th March  2021

01895 277986