EFAF Energy Statement - Rosedale College - EFAF Temporary Modular Teaching Accommodation

GENERAL & EMERGENCY LIGHTING

A maintained level of LED  lighting  shall be provided and shall be designed to meet the Lighting Requirements of Approved Document Part L. Lighting to main areas will be controlled by PIRs for energy efficiency.

The emergency lighting system shall be designed and installed in accordance with current relevant British Standards and CIBSE TM12 recommendations. Lighting will generally utilise An average  4.7 w/m2 subject to design and spec.

HEATING

Generally heating will be Via AC but any electric panel / down flow heaters will be controlled by thermostat and time clocks

Fujitsu DX inverter driven units, ceiling mount and wall mounted air conditioning systems, controlled with hand-held infra-red remotes.

Inverter air conditioners use an inverter-controlled compressor, that adjusts its speed to meet the desired temperature, resulting in a variety of benefits such as lower energy usage, more consistent cooling and heating, and a longer lifespan than traditional air conditioning units.

Fujitsu systems can be set on heating, cooling, or auto mode to achieve a comfortable working environment.

Fujitsu units are environmentally friendly, heavily investing in less harmful refrigerants to the integration of inverter technology, these systems are designed to use less energy. Lower power consumption also means fewer Co2 greenhouse gas emissions been set into the atmosphere, coupled with the most ozone friendly R410A + R32 Refrigerants.

Non-use of gas for heating.