12 WIELAND ROAD

NORTHWOOD

HA6 3QU

SEQUENCE OF DEVELOPMENT.

Client: Ms. Shoma Gutpa

Planning consultant : Keith Dunford

Main contractor: AP Builders Ltd

Site Manager; Tajinder Singh

The site is open fronted to the road with the rear of the property screened off by trees and fences. There is no access the rear without a key. Contractor provided with the key and no goods left to the front accept the temporary toilet and skip. Exiting patio doors to the terrace to be locked and a warning tape of ‘do not open’ placed across them.

- Site mobilisation to include Porta Loo in front driveway, area of material storage to be side of house behind side fence (not visible from road) and skip on front driveway; not on road. Warning signs to be placed.

- Erection of tree protection fence minimum 1.5M high

- Place boards 18mm thick to all areas where near demolition and for the ease of wheeling barrows.

- Demolition to start with the removal of existing patio slabs ( put aside for re-use should the client desire)  and the top of the retaining wall to a level where the wall appears stable.

- Commence digging of footings, inspecting retaining wall for stability.

- Pour concrete in footings. Concrete to be ready mixed and pumped into place.

- Build up new retaining walls in concrete block in accordance with good building practise.

- Allow for drainage and weep holes

- Pre-cast concrete beams to be delivered to road side and immediately taken to site with no materials left on the road unattended or overnight.

- Beams man hauled into place with appropriate blocks.

- Inspection made as to the required back fill using existing demolished materials.

- Drainage gullies positioned.

- Steps formed on the block and beams.

- Patio slabs to clients requirements laid and pointed.

Risks Involved

- All personnel and visitors to wear minimum, high vis vests, helmet, gloves and steel capped boots. Eye goggles to be worn at appropriate times; e.g. cutting of blocks.

- Carrying materials, risk of dropping medium, demolition debris falling risk low, excavation trench clasping risk low.