PLANNING DESIGN AND ACCESS STATEMENT

Site:	6 Great Central Avenue, Ruislip HA4 6TR

Proposal: 	Conversion of previously extended bungalow to three self-contained flats.

Local Authority:	LB Hillingdon

Prepared by:

ABA Chartered Surveyors

49 The Green

Southall

Middlesex

UB1 2AR

Date: 08/12/2021

- Introduction

This design and access statement is submitted to support the accompanying planning application to convert the existing, previously extended bungalow into three self-contained flats.

- Location

The application site is located on the north end of Great Central Avenue near to the junction with Station Approach and virtually across the road from the South Ruislip Underground station.

3. Site Analysis and Evaluation

The application property is a detached bungalow, with the loft space recently converted to a habitable use with dormer extensions on the sides of each roof slope. The front of the premises is currently hard surfaced and used for parking and bin storage.

The prevailing character of Great Central Avenue and surrounding area is suburban residential with two storey semi-detached properties with bay windows and detached and semi-detached bungalows mixed in. Most of the front gardens have been paved to provide off street parking. Immediately opposite the property is a large 3 storey office building.

4. Proposed Development

It is proposed to convert the previously extended house into three separate self-contained flats. The existing premises has sufficient accommodation to the allow conversion/ adaptation without any further extensions.

5. Principle of conversion

The existing extended dwelling has a floor area of 165m2 and would contain a 0ne bedroom flat at first floor level with a bedsit dwelling on the ground floor at the front, and one bedroom flat on the ground floor at the rear and would have direct access to a garden which would be in excess of the 50m2 required. The principle of the conversion into flats is therefore satisfied.

6. Schedule of Accommodation

The proposed development will provide the following accommodation:

| PROPOSED | No. of Bedrooms | No. of Habitable Rooms | Potential Accommodation | Floor Area (m2) | London Plan floor area standard (m2) |
| --- | --- | --- | --- | --- | --- |
| Ground floor flat | bedsit | 1 | 2 | 39 | 39 |
| Ground floor flat First floor flat | 1 1 | 2 2 | 2 2 | 53 56 | 50 50 |

The individual room sizes are annotated on the drawing accompanying this application.

7. Layout and Design Strategy

The form and bulk of the existing building will not be affected by the proposed development.

All habitable rooms would be served by openable, clear glazed windows which would be front and rear facing which would allow for good levels of light, outlook, and ventilation. The overall living conditions of the proposed units are of a good standard.

The internal circulation spaces and staircases have been designed to provide adequate space for normal use, including movement of normal household furniture.

8. Access

The new front door will have ramped/ level access, suitable for a wheelchair, from the street level. The internal layout will comply with the guidance contained in Part M of the building regulations, as far as possible within the constraints of an existing building.

8.1. Public transport and parking

As the site has good access to public transport services with South Ruislip underground station within 50m and easy 2 minute walk to various bus routes. It is proposed to retain the one existing parking space.

8.3. Cycle Store

The London Plan requires two secure bicycle parking spaces for each flat, which will be provide in secure lockers in each private rear garden area.

9. Refuse Storage

Bin storage is also shown in the front garden of the site, allowing easy collection from Great Central Avenue.

10. Impact on Neighbouring Properties

No enlargement of the property is proposed and as the dwelling would remain in residential use it would not have a detrimental impact on the occupants of neighbouring properties. Whilst a bin store is proposed to the front of the dwelling, it would be located in the front garden area as existing. It would be located away from any habitable room windows to ensure no detrimental impact on the occupants of the application site or the neighbouring properties.

11. Sustainability

The proposed development will be designed to include the sustainable features listed below:

- New construction will be fabricated with sustainable materials wherever possible, which will be easy to maintain.

- New boilers/ heating systems will be most energy efficient available.

- Timber will come from approved and sustainable sources as approved by FSC.

- All light fittings will be low energy.

- The site is located with the North Greenford shopping and commercial centre with all the amenities within a walking distance and public transport links to further afield.

12. Conclusion

The proposed development will make more efficient use of previously developed land. The premises will now be more sustainable as three smaller units providing self-contained accommodation for a similar number of occupants. We therefore hope the Local Planning Authority will be able to consider the application favourably. However, we would appreciate an opportunity to discuss and amend the design if that is considered appropriate, before the application is decided.