Sandmore House

14 Sandy Lodge Way

Northwood

HA6 2AJ

Tree Number - T1

Tree Type - Oak (Quercus robur)

Approx. Diameter at 1.5m - 0.5-0.75m

Approx. Height - 18m

Approx. Crown Spread Diameter - 16m

Location - Rear garden Rear boundary

Service - Crown Reduction

OPTION 2 - Going ahead with this option

Work Required - Crown reduce back to previous  by removing between 2-3m from the height and between 1.5-2.5m from the sides

Reason - as part of continued maintenance to control and contain overall size of tree

Priority Code - on

Tree Number - T2

Tree Type - Yew (taxus baccatta)

Approx. Diameter at 1.5m - 0.1-0.3m

Approx. Height - 7m

Approx. Crown Spread Diameter - 7m

Location - neighbours tree no. 12 Front garden left hand boundary

Service - prune

Work Required - cut back to boundary fence all overhang to the clients garden by removing upto 2m

Reason - as per clients wishes to reduce encroachment over clients garden and there young cherry trees

Priority Code – on

Tree Number - G1

Tree Type - x3 Cherry (prunus spp)

Approx. Diameter at 1.5m - below 100mm

Approx. Height - 6-7m

Approx. Crown Spread Diameter - 3-4m

Location - Front garden right hand side

Service - Crown Reduction

Work Required - Crown reduce back to previous  by removing between 1-1.5m from the height and between 0.5-1m from the sides

Reason - as part of continued maintenance to control and contain overall size of tree

Priority Code – on

Tree Number - T3

Tree Type - Cypress (Cupressus spp.)

Approx. Diameter at 1.5m - below 75mm

Approx. Height - 5m

Approx. Crown Spread Diameter - 1m

Location - Front garden by building right hand side

Service - Fell and grind

Work Required - Fell and Grind out the stump to a minimum of 200mm below the stump's current ground level and leave all stump arising as in-fill. Please note in-fill stump arisings will be mounded and be left above ground level to allow for settling

Reason - as per clients wishes to allow more light to the apartments

Priority Code – on

Tree Number - T4

Tree Type - Beech (Fagus sylvatica)

Approx Diameter at 1.5m - 0.1-0.3m

Approx Height - 12m

Location -  Front garden left hand side

Service - Dead wood

Work required - Remove all deadwood over 40mm in diameter and/or 1m in length

Reason - to remove potential hazards

Priority code – B

Tree Number - T5

Tree Type - Plum (prunus spp)

Approx. Diameter at 1.5m - 0.3-0.5m

Approx. Height - 7m

Approx. Crown Spread Diameter - 6m

Location - Front garden left hand side

Service - Crown Reduction

Work Required - Crown reduce by removing around 1m from the height, remove around 2-2.5m from the overextending limb and remove around 0.5-1m from the remaining sides to shape

Reason - to Re-balance balance appearance and take end weight off overextending limb

PrioTree Number - T6

Tree Type - Cherry (prunus spp)

Approx. Diameter at 1.5m - 0.1-0.3m

Approx. Height - 4.5m

Approx. Crown Spread Diameter - 3m

Location - Front garden Front boundary right hand side

Service - Crown Reduction

Work Required - selectively crown reduce by pruning in the 3-4 overextending branches by removing around 0.5m to bring back in line with remaining crown

Reason - as per clients wishes to re-balance appearance

Priority Code - onrity Code – C