Front Door; Main roof; Bricks on front elevation; Fascia Boards and Soffits; Rainwater Pipes and Gutters; and Driveway

As existing. Rainwater goods are black plastic.

Bricks for Extension

Red brick (per approved plans). Berkshire Red from Taylor Maxwell.

Dormer Cheeks/Roof/Capping Details

Brown Zinc (per approved plans). Pigmento Brown from VM Zinc.

Windows to front and side elevation

White Sash Timber Woodgrain Windows (per approved plans)

Windows and doors to rear elevation

Brown aluminum to match zinc cladding per approved plans. Alumi-lite SLC casement windows and Alumi-lite SLD – Side Hung Slimline Doors from L2i Limited.

Balustrade to balcony on front elevation

Black powder coated metal to match neighboring houses (image shown from 20 Murray Road)

Balustrade to balcony on rear elevation

Clear 17.5mm laminated toughened glass with clamp fittings (per approved plans)

First Floor Roof

Wildflower Green Roof (per approved plans)

Second Floor Roof

Single ply membrane (per approved plans)

Rooflights

Conversation type set flush along the roof line (per approved plans and officers’ report)

Balcony

Brown/Grey Decking (per approved plans)

Privacy Screen/Planting

Brown timber privacy screen with green laurel planting – Minimum height of 1.8m from the finished floor level (per approved plans)