Mr Harnek Sidhu

31 Shakespeare Avenue

Hayes

Middlesex

UB4 0BB

25th March 2022,

London Borough of Hillingdon

Planning & Community Services

3N, Civic Centre

High Street

Uxbridge

UB8 1UW

Dear Sir,

Re: Shakespeare Avenue, Hayes, UB4 0BB

I attach herewith Planning application at the above premises, and would like you to consider the following in support of the application.

Design / Access Statement.

The proposal

The proposal seeks the alterations and conversion of dwelling at 31 Shakespear avenue, Hayes in to 2 bed 4-person self-contained flats at ground floor level (number 31), and 2 bed 3 person self-contained flat at first floor and loft floor level (number 31A). The residential proposal is in accordance with the London Plan Consolidated 2015 standards on internal space, layout and proportion of wheelchair adaptable units. The London Plan requires internal floor space of 70.0m2, the proposed ground floor flat provides 78.0m2, the first and loft floor flat provides 76.5m2 and required is 70.0m2.

The Site

The existing house is a single-family dwelling house of the early 20th Century origin. The site is semi-detached property with front forecourt. The existing accommodation comprises of three bedrooms, and bath/wc at first floor level, one bedroom with en-suite at loft floor level and lounge, dining room, shower/wc, study and kitchen at ground floor level. The house has a 7.0m deep rear garden after the rear extension. The dwelling is located within an established residential area and many of the surrounding properties have been altered/converted in to flats or extended over time.

There is a large store outbuilding 5.5m deep at the rear of the garden.

Planning Policy

Policy relevant to the application is contained in the Development Plan Policies within Hillingdon Council’s Local Plan: Part 1, Saved Unitary Development Plan policies, the London Plan 2015, NPPF and supplementary planning guidance prepared by both LB of Hillingdon and the GLA. Also of relevance is the Supplementary Planning Guidance, entitled ‘extensions and Householders Guide’ and ‘Accessible Homes’.

Design

The original house is considered to be of average size for a modern single-family dwelling. The current proposal has sought to minimise the impact of the proposed conversion by not proposing any major changes to the external elevations.

The conversion of the dwelling will conform to the requirements of Accessible Homes. The Ground floor design incorporates the level access entrance approach at front of the premises, doorways of minimum width of 800mm, and 1500mm turning circle for wheel chair circulation. A wider car parking space is provided for the ground floor flat at the front forecourt measuring 3.3m wide x 4.8m deep (see drawing number 09).

The proposal is not likely to have greater impact on the adjoining premises than its current use. The whole scheme provides the need for growing demand for smaller residential units in the borough. Provision for a rear garden space is also provided to ground floor flat. There is Regional open space park (Belmore playing fields) nearby at the end of Shakespeare Avenue, for the enjoyment of the first floor flat (31A). Cycle stand/storage facilities are to be provided for both the flats.

Provision for the waste storage bins is provided at the front of the house, see drawing number 09.

Access

There are no major access implications with this proposal as it is merely a conversion of an existing property in an established residential area. There is also wide range of shops and facilities on the main Uxbridge road, within easy reach via the public transport and on foot. In terms of the disabled access, the ground floor of the new dwelling flat is capable of being accessed by persons using a wheelchair via the ramped entrance.

I trust the above is satisfactory for you to grant approval. However if you require any further information please do not hesitate to contact me.

Yours faithfully

H Sidhu