DESIGN AND ACCESS STATEMENT.

43 FRITHWOOD AVENUE, NORTHWOOD, MIDDLESEX.

Assessment.

The property is a detached house on a wide fronted plot on the East side of Frithwood Avenue. The house is built in facing brickwork with a plain clay tiled roof.

There is a gravel carriage driveway up to a centrally located front entrance door.

There are two mature Cedar trees (TPO 149: T12 and T14) in the front central lawn with a low hedge bordering the pavement.

Proposal.

It is proposed to construct entrance gates to the carriage driveway and plant a new low hedge to the frontage.

A planning application (74380/APP/2021/2419) for entrance gates comprising brick piers with stone cappings and metal gates was refused consent on the 25th August 2021 due to the siting of the entrance gates and the height of the proposed hedge along the frontage to the detriment of the highway and pedestrian safety.

This new planning application is also for facing brickwork piers with reconstituted stone cappings and metal gates together with new frontage hedging.

This planning application addresses both reasons for refusal by setting the entrance gate piers 4800mm back from the kerb line and 2600mm from back of the pavement line and proposing a new low maximum 500mm high hedge of Euonymus Japonica bordering the street frontage.

The revised design scheme provides for good left and right hand visibility splays from the carriage drive entrances, with the setting back of the entrance piers and gates and the proposed new low hedging, which will ensure good highway and pedestrian safety.

If the proposed design scheme is considered acceptable then an arboricultural and construction method statement could be submitted for pre-commencement conditional approval.

October 2021.