- Surface Water/SUDS Maintenance Statement – ref 12003

- Regularly inspection of the surface water drainage network for blockages and clearing unwanted debris / silt from the system should improve the performance of the surface water network and decrease the need for future repairs. In the event of blockages, high pressure water jets can be used to clear the gullies and pipes to ensure they are functioning correctly, this should be undertaken by certified trained professionals.

- The level and frequency of maintenance required on site is dependent on the type of facility. The type of maintenance will fall into one of three categories “regular maintenance”, “occasional maintenance” and “remedial maintenance”.

- Regular maintenance of the drainage and SuDS features will include, inspections, removal of litter / debris and sweeping of the surfaces. Occasional maintenance will include removal of sediment etc. and remedial maintenance may include structural repairs and infiltration reconditioning if required.

- The drainage and SuDS elements after an initial inspection following construction should be inspected on a monthly basis for the first 12 months and after large storms, thereafter the following maintenance regime should be applied and adjusted if the 12-month monitoring process has identified any issues.

- Following completion of the development it will be the freeholder responsibility to maintain the drainage network, including the SuDS elements.

- Attenuation Tanks

- For the attenuation tanks, the following maintenance is recommended.

| Attenuation Tank Maintenance Schedule |  |  |
| --- | --- | --- |
|  | Required Action | Typical Frequency |
| Regular maintenance | Inspect and identify any areas that are not operating correctly. If required take remedial action | Annually |
|  | Remove debris from the catchment surface (where it may cause risk to performance). | Monthly |
|  | For systems where rainfall infiltrates in the tank from above, check surface of filter for blockage by sediment, algae or other matter, remove and replace surface infiltration medium as necessary | Annually |
|  | Remove sediment from pre-treatment structures. | Annually or as required |
| Remedial Actions | Repair/rehabilitate inlets/outlets, overflows and vents. | As required |
| Monitoring | Inspect/check all inlets, outlets, vents and overflows to ensure that they are in good condition and operating as designed. | Annually |
|  | Survey inside of tanks for sediment build-up and remove if necessary | Every 5 years or as required |

Table 8.1 – Attenuation Tank Maintenance