DAVID  IAN  ANDREWS

Tec.Dip., B.Sc. (Hons), MRICS.

CHARTERED  BUILDING  SURVEYOR

# Scarthoe, Market Street, Knighton, Powys, Wales, LD7 1EY

# Tel. 01547 520954

# Mob. 0797 965 0629

E-Mail: davidandrewsmrics@gmail.com

Planning Specification

To accompany planning application

In respect of

Proposed conversion of garage within existing dwelling house

At

150 Hoylake Crescent, Ickenham, Middx. UB10 8JJ.

For

Mr Pradeep Kotecha.

External walls

Infilling to the lower portion of the existing up & over garage door opening:

Facing brickwork matching the existing brickwork piers each side.  New work toothed into existing to avoid unsightly continuous vertical mortar joint.

Roof slopes

No changes.

Window frame

Mullion and transom lines as per elevation.  Dummy casements provided to ensure equal sight lines.  Frames to be white factory finish aluminium in hardwood sub frames to match existing front elevation windows.  Sub-frames to have dark brown stain finish – Sadolins or similar product – so as to match existing sub-frames.   Glazing to have diamond configuration mock leading to match existing windows.

Window sub-sill

To be constructed of clay plain creasing tiles, configured to match existing first floor and ground floor bay windows.

Jan. 2021

David Andrews MRICS

Chartered Building Surveyor.